# EXERCISE NO. : 8

# CLOSED LOOP PNEUMATICS

# STATUS CONTROLLER

DATE:

Reg. No. :

PREREQUISITE KNOWLEDGE:

- Fundamentals of pneumatics and its basic components.

- Fundamentals of FluidSIM software for simulation of pneumatic and electrical circuits.

OBJECTIVES:

  - To be able to set the parameters of a status controller

  - To get familiarized with the influence of controller coefficients

THEORY:

5/3 Proportional Valve

Linear Potentiometer

Linear Axis (Rod-less Cylinder)

Status Controller

Festo Status Controller

TASK 1 – POSITION CONTROL OF A PNEUMATIC CYLINDER IN A DEBURRING APPLICATION

Assembly and Commissioning of Closed Loop Circuit

Investigation of Transient Response for Various Coefficients

Parameterization of the Controller Using Empirical Method

Use the following specifications for the rod-less cylinder

| Specification | Value |
| --- | --- |
| Type | Double acting cylinder, Piston rod-less cylinder with slide, Adjustable Cushioning |
| Stroke | 450mm |
| Piston Diameter | 25mm |

DELIVERABLES

TASK 1

- Pneumatic and Electrical circuit diagrams

| (a) Pneumatic Circuit Diagram | (b) Electrical Circuit Diagram |
| --- | --- |

- Investigation of the transient response for various coefficients.

  - Present the state diagram of the controlled variable for the following cases to understand the influence of the respective coefficients.

  - Case 1:

Case 1.1 [Kx= 0.4, Kx= 0.0,Kx= 0.0,P= 8.5] – Overshoot with one forward swing

Case 1.2 [Kx= 0.7, Kx= 0.0,Kx= 0.0,P= 8.5] – Overshoot with two forward swings

  - Case 2:

Case 2.1 [Kx= 1.0, Kx= 2.5,Kx= 0.0,P= 8.5] – Overshoot with two forward swings

Case 2.2 [Kx= 1.0, Kx= 7.0,Kx= 0.0,P= 8.5] – Overshoot with slight forward swing

  - Case 3:

Case 3 [Kx= 0.4, Kx= 0.0,Kx= 10,P= 8.5] – Overshoot with one forward swing

- Parameterization of status controller using the empirical method. Note the values which have been set for the characteristic data to obtain a minimum or no overshoot, minimum steady-state error and quick settling time.

| Characteristic Data | Values |
| --- | --- |
| Overall Gain | 25 |
| Position Coefficient | 1.8 |
| Velocity Coefficient | 27 |
| Acceleration coefficient | 0.1 |

- Present the state diagram of the generator, displacement encoder and position of the piston.

LAB SESSION SCREENSHOT

INFERENCE

The working of a proportional valve was studied from both theoretical and practical viewpoints. A closed loop electro-pneumatic circuit to control the position of a double-acting rodless cylinder was constructed and simulated using Festo FluidSIM.

The voltage setpoint to the solenoid of the proportional DCV was a square wave of frequency = 0.5 Hz, amplitude = 1 V, and offset = 5 V, to control the valve spool proportionately about its mean position (@5V). The signal generator voltage (input) was not directly comparable against the piston position of the double-acting rodless cylinder as the physical units of these two quantities were not identical. Nonetheless, the displacement encoder (linear potentiometer) output in terms of voltage was directly comparable against the signal generator voltage, since the two had same voltage scale.

The theory of status controller was understood and a simulated status controller was implemented to control the position of a double-acting rodless cylinder. The effect of position, velocity and acceleration coefficients Kx, Kx and Kx respectively was analysed by observing the transient response of the controlled variable (piston position of the double-acting rodless cylinder). Finally, the coefficients were tuned to get minimum (or no) overshoot, minimum steady-state error and quick settling time. The coefficient values were reported along with the corresponding state diagrams for signal generator (input), displacement encoder (feedback) and position of the piston (output).